# WLAN installation in Linux

Preparing Linux box for WLAN operation in most cases are quite straightforward. In the case the latest Linux release is used, it is most likely to support your old WLAN cards, especially, Orinoco cards, and will automatically detect and load the card’s driver.

I am using Linux Mandrake most of the time, mainly due to its easiness in the installation & configuration of the system. Since most of the applications are readily in RPM format, it would be very easy to install software in Linux Mandrake. I would strongly recommend using Linux for your 24 hours wireless Internet gateway.

## Preparation Stage

- Prepare the computer that will be used as 24 hours wireless gateway to the Internet. Computer specification used at my home gateway is quite low, i.e.,

  - Pentium II 166MHz, 64MB RAM, HardDisk 3Gbyte.

  - Ethernet (LAN Card) with sufficient UTP cables.

  - Hub 10Mbps (I am using used hub, cost me about US$3-5).

  - Monitor, not necessary a good one. I am using an old S3 2Mbyte RAM video card

  - CDROM drive (I am using 8x CD drive).

- Install the 2.4GHz antenna as well as its coaxial cable

- Prepare the WLAN card. I normally not insert the WLAN card during the software installation processes.

- I would like to suggest to use WLAN card that supported by PCMCIA driver in Linux. Summary of the WLAN card supported by PCMCIA driver in Linux last updated 2001/03/09 01:19:25 is listed below.

- If the card is not supported by PCMCIA, you may want to check several places that may carry the driver, such as,

  - http://www.magiclan.com (the official place of Samsung SWL 2000P).

  - http://www.linux-wlan.com

  - http://www.hpl.hp.com/personal/Jean_Tourrilhes/Linux/Linux.Wireless.drivers.html

  - http://www.hpl.hp.com/personal/Jean_Tourrilhes/Linux/Tools.html

- Coordinate with your ISP to get the required information on:

  - ESSID

  - IP address of our PC / WLAN card

  - IP address gateway at the ISP.

  - Channel / Frequency.

  - DNS Server.

## Linux WLAN Supported Device List

Linux WLAN is normally driven as a PCMCIA device, the driver is maintained by David Hinds dahinds@users.sourceforge.net. This list is last updated: 2001/03/09 01:19:25. You may find the newer one from www.sourceforge.net.

Wireless network adapters that supported by Linux PCMCIA are,

[airo_cs driver] [x86]

Aironet PC4500, PC4800

Cisco 340

Xircom Wireless Ethernet Adapter

[netwave_cs driver] [x86]

Breezenet SA-PX

Xircom CreditCard Netwave

[ray_cs driver] [x86,axp]

BUSlink Wireless LAN Adapter

Raytheon Raylink

WebGear Aviator 2.4, Aviator Pro

[wavelan_cs driver] [x86,smp]

AT&T / NCR / Lucent WaveLAN version 2.0

DEC RoamAbout/DS

[wvlan_cs driver] [x86,axp,ppc,smp]

1stWave 1ST-PC-DSS11IS, DSS11IG, DSS11ES, DSS11EG

ARtem Onair ComCard STD & EMB versions, 128- & 64-bit

Cabletron/Enterasys RoamAbout 802.11 DS

ELSA AirLancer MC-11

IBM High Rate Wireless LAN

Lucent Orinoco WaveLAN/IEEE 802.11(b)

Melco WLI-PCM-L11

NCR WaveLAN/IEEE 802.11

[ PrismII based cards: limited functionality ]

Addtron AWP-100

Ambicom WL1100 PC

Compaq WL100

D-Link DWL-650

Linksys WPC11 Instant Wireless

SMC2632W

For maintenance reasons, some cards are not distributed within the base PCMCIA package. Most of them are still available on the Linux PCMCIA FTP site, at ftp://pcmcia-cs.sourceforge.net/pub/pcmcia-cs/contrib,

[asplus_cs driver]

Netwave AirSurfer Plus wireless network adapter

http://ipoint.vlsi.uiuc.edu/wireless/asplus.html

(Jay Moorman <jrmoorma@uiuc.edu>)

[brzcom_cs driver]

BreezeCOM BreezeNet SA-PCR Pro.11 Series wireless adapter

http://www.breezecom.com, support@breezecom.com

[spectrum24t_cs]

3Com 3CRWE737A AirConnect

Intel PRO/Wireless

Symbol Technologies Spectrum24 11 Mbps wireless adapter

(Tim Gardner <timg@tpi.com>)

[wavelan2_cs driver]

Lucent WaveLAN/IEEE wireless network adapter

http://www.wavelan.com

(Lucent Technologies <betasupport@wavelan.com>)

Some cards have their own web sites.

[ Intersil PrismI wireless cards ]

Harris PRISM/AM79C930 IEEE 802.11 wireless LAN

Nokia/InTalk ST-500A

Nokia C020

Samsung MagicWave SWL-1000N

Zoom Telephonics ZoomAir 4000

http://www.absoval.com/linux-wlan

(Mark Mathews <mark@absoval.com>)

[ Intersil PrismII wireless cards ]

Addtron AWP-100

Ambicom WL1100 PC

Compaq WL100

D-Link DWL-650

Linksys WPC11 Instant Wireless

Samsung MagicLAN

SMC2632W

http://www.magiclan.com

http://www.absoval.com/linux-wlan

Proxim RangeLAN2 and Symphony wireless LAN cards

http://www.komacke.com/distribution.html

(Dave Koberstein <davek@komacke.com>)

## Linux Installation of PC Gateway

- Nothing really special in installing Linux for 24 hours wireless gateway. Some security concern and minimal installation may be needed.

- I use the latest version of Linux Mandrake for most of my installation, as it is very easy to do and, thus, minimize my effort in educating others in using Linux.

- Since in most cases the gateway must provide the required support for its IntraNet, I normally install several required software, such as, wireless-tools, pcmcia-cs, iproute2, fetchmail, iptables, squid, postfix. If less supported card is used, development package and kernel-pcmcia may need to be installed for compiling the driver if needed.

- On a Pentium II 166MHz with 8x CD drive machine, we normally need approximately 45-60 minutes to complete the installation processes.

- After the completion of Linux installation processes, the local Ethernet card (UTP LAN) is activated and given the IP address of 92.168.0.1 to make it easy in performing Internet Connection Sharing.

- After the Linux installation processes, the WLAN card can now be inserted. If a common WLAN card is used, the operating system will normally detect the card & install the required driver. We may than configure the network for the card.

## Configuring the WLAN

- A bit different than Windows environment with Graphical User Interface (GUI), in Linux, the configuration is performed in text mode. Most of the files needed to configure your WLAN are located at /etc/pcmcia. The needed files are config.opts, network.opts & wireless.opts. Note that some of the parameters in these three (3) files can be overlapped.

- Edit /etc/pcmcia/config.opts. Example of parameters to be added, can be,

  - For MAN/WAN connected to an Access Point (AP)/ISP

module “swldpc11_cs” opts “networkmode=1 essid=ispnet”

  - To increase transmission reliability, sometime it may help by reducing the MTU & increasing Access Point Density.

module “swldpc11_cs” opts “mtu=500 apdensity=3”

The complete parameters are listed below.

- Edit file /etc/pcmcia/network.opts. Configure IP address, gateway & DNS ISP, e.g.,  (please confirm it with your ISP)

*,*,*,*)

IF_PORT=""

BOOTP="n"

IPADDR="10.0.0.5"

NETMASK="255.255.255.0"

NETWORK="10.0.0.0"

BROADCAST="10.0.0.255"

GATEWAY="10.0.0.1"

DOMAIN="domain.org"

DNS_1="dns1.domain.org"

;;

- Edit file /etc/pcmcia/wireless.opts to configure the radio network. Please note that some parameters can be set in /etc/pcmcia/config.opts). An example is:

*,*,*,*)

INFO="SAMSUNG 11Mbps WLAN"

#ESSID (extended network name) : My Network, any

ESSID="test3"

# Operation mode : Ad-Hoc, Managed

MODE="Managed"

# Frequency or channel : 1, 2 (channel);2.422G, 2.46G (frequency)

CHANNEL="4"

# Bit rate : auto, 1M, 11M

RATE="auto"

# Encryption key : 4567-89AB-CD, s:password

KEY="883e-aa67-21 [1] key 5501-d0da-87 [2] key 91f5-3368-6b [3] key 2d73-31b7-96 [4]"

# Other iwconfig parameters : power off, ap 01:23:45:67:89:AB

IWCONFIG="power on"

;;

- Competing the editing of these three (3) files, you basically completed the configuration processes.

- Turn off the PC gateway. Insert the WLAN card into the existing PCI slot, and connect the external antenna.

- Please note that it would be safer to turn on the WLAN card with the antenna attached to reduce any reflection RF power that may ruin your card.

## A Complete List of Module Parameter

| networkmode | Select 802.11 network type 1 = Infrastructure mode (default) 3 = Ad-hoc Mode |
| --- | --- |
| stationname | Station name |
| essid | Network name (SSID) In ad-hoc mode, this will be the name of independent network. If Access Point (AP) is used, ESSID must be set the same as the ESSID used at AP. |
| txrate | Configure data transmit speed 1 = fixed 1Mbps 2 = fixed 2Mbps 3 = auto select 1 or 2 4 = fixed 5.5Mbps 8 = fixed 11Mbps 15 = Fully Auto(default) |
| psmode | Power Saving Mode (if this mode enabled, throughput may drop) 0 = disabled(default) 1 = enabled |
| channel | Channel used in ad-hoc mode. 4 = default In most models, channel number 1-13 can be used. |
| apdensity | Access Point density (sensitivity) Set Access Point density, it may affect roaming capability of the workstation with WLAN card. 1 = low density (default) 2 = medium density 3 = high density |
| threshold | Medium reservation (RTS/CTS frame length) 500 = hidden stations 2347 = no RTS/CTS (default) valid range: 0-2347 |
| fragthresh | Define total byte to be sent in fragmentation process. 2346 = default range: 256-2346 (even number only!) |
| mtu | Maximum transfer unit 1500 = default valid range: 256-2296 |

## Some troubleshooting tips

With the WLAN card inserted into the slot, when Linux is turned on, the card manager will give two beeps to indicate its success / failure during the configuration processes.

- Two high tone beeps.

Card has been successfully identified & configured.

- High tone in the beginning followed by low tone.

Card has been identified, failure in configuration. Might be helpful to see /var/log messages to see mistakes in configuration.

- Single low tone.

Failure in identifying the card. Run “cardctl ident” to see the information of the attached PnP adapters.

## Monitoring WLAN Performance

To monitor WLAN performance, we use iwconfig provided in wireless-tools, such as

# iwconfig eth0

An example of the output is as follows

eth0      SAMSUNG WLAN  ESSID:"ISPNET"  Nickname:"warok"

Channel:6  Sensitivity:3/3  Mode:Managed

Access Point: 00:40:05:DE:27:EC

Bit Rate=11Mb/s   RTS thr=150 B   Fragment thr=2346 B

Encryption key:off

Power Management:off

Link quality:0/92  Signal level:27/153  Noise level:0/153

Rx invalid nwid:0  invalid crypt:0  invalid misc:0

Some of the point of interest are, Signal level (the higher the better), Link quality (the higher the better) & Noise level (the lower the better). Bit Rate might of interest especially if automatic speed is used.

For those who like to browse http://www.sourceforge.net, you might find a large collection of tools for monitoring your wireless equipments. I normally run Multi Router Traffic Grapher (MRTG) and IP Accounting Next Generation (ipac-ng) for monitoring my wireless gateway.

## Compiling & Installing WLAN Driver

Those who are unlucky and use WLAN card that comes with no driver in the core PCMCIA-CS. We may need to compile and install the driver rather manually. I am using SAMSUNG SWL 2000P WLAN Card at home and have to compile the driver in Linux Mandrake 8.0.

SAMSUNG provides two (2) drivers, namely swld11_cs-1.21.tar.gz (for PCMCIA card) & swldpc11_1.21.tar.gz (for PCI card). I am using the PCI card. The compiling and installation processes are as follows,

- Those who use Linux Mandrake 8.0 & installed kernel-pcmcia during installation processes, there is no need to install David Hind PCMCIA package source from http://pcmcia-cs.sourceforge.net/. You may find your kernel-pcmcia under directory /usr/src/linux/pcmcia-cs-x.x.xx.

- Copy the SWL 2000P driver under /usr/src/linux/pcmcia-cs-x.x.xx.

- Untar the driver using

#  tar zxvf swldpc11_1.21.tar.gz

- Configure the source

# make config

some parameters to be set during the installation processes are

  - Linux source directory is /usr/src/linux

  - “Kernel specific option” may be the most critical parameter to be picked. It gives two (2) options, namely, [1] Read from Currently Running Kernel or [2] Read from the Linux source tree.  I normally pick [1].

- Since the original driver is written for kernel 2.2.x, and Linux Mandrake 8.0 is running on kernel 2.4.x. We need to do some hacking,

  - Change to directory /usr/src/linux/pcmcia-cs-3.1.25/clients/

  - Edit swldpc11_cs.mk

  - On the third (3) sentence from below @mkdir –p .depfiles; etc …

  - Put remarks “#” in front of the sentence.

  - Edit file swldpc11_cs.c.

  - Find MODULE_PARM(khannel,”i”);

  - Change into MODULE_PARM(channel,”i”);

- Change to directory /usr/src/linux/pcmcia-cs-3.1.25. Compile & Install driver SWL 2000P, with:

# make all

# make install

- At this point, the installation of SWL 2000P driver has been completed, and we can continue to configure the card as previously described.