Computer Networking in Indonesia:

Current Status and Recommendations

for its developments

Soegiardjo Soegijoko, Onno W. Purbo, Widiadnyana Merati

Priyono Sutikno, Intan Achmad.

Institute of Technology Bandung

Bandung 40132, Indonesia

FAX +62-22 2503253, 214417, 435226

soegi@elka.ee.itb.ac.id, cnrg@cnrg.itb.ac.id, yc1dav@itb.ac.id

yc1dav@garuda.drn.go.id, intan@melsa.net.id, lpitb@melsa.net.id

January 1996

Abstract

This report is an effort to identify various networking initiatives, current status & major recommendations. A significant increase in the Indonesian networking infrastructure as well as number of users is apparent in the 1995. We are looking at a total user of close to 20,000 in 1995. A total of sixteen (16) ISPs have received their licensed from the Indonesian government. Currently (end 1995) the total speed to Internet is 640Kbps, we are hoping to increase the speed to close to 5,4Mbps by the end 1996. ATM, B-ISDN & Frame Relay 2Mbps regional / nation wide backbone is currently in implementation & hope to be in operation by the end of 1996. Several TCP/IP-based VSAT networks are in implementation. A neutral hub among Indonesian ISPs is being proposed by ITB. Some of the activities & recommendations will reflect ITB’s commitment & view towards computer networking in Indonesia. The authors are not claiming the report to be complete as the computer networking development in Indonesia is too rapid and too distributed to keep up.

Introduction

The status of computer network in Indonesia as of June 1995 is presented in http://www.ee.itb.ac.id/~yc1dav/indo-net.asc.  The estimated user composition of the network is shown in Figure 1.

Figure 1. Estimated Indonesian Internet User Composition

(June 1995)

Within 12 months, there is a siginificant shift in the user composition from mostly educational institution (60% of the users) to mostly commercial users (42%) in June 1995.

Figure 2. A Simplified Network Topology & Plan To Setup An ITB’s Neutral Hub

(January 1996)

It is interesting to see that by the end of 1996, Indonesia is expected to have at least 5,4Mbps access to Internet. There are currently several major gateways to Internet from Indonesia as listed in Table 1 below.

Table 1 List of Major Internet Gateways

| Name | Location | E-mail | Connection | Speed |
| --- | --- | --- | --- | --- |
| IPTEK-NET | Jakarta | ?@iptek.net.id | 3 other cities | 64Kbps |
| IndoNet | Jakarta | sanjaya@indo.net.id | 5 other cities | 128Kbps to SingNet |
| RadNet | Jakarta | support@rad.net.id | ? | 512Kbps |
| Sistelindo | Jakarta | ?@ibm.net | 2 other cities | 128Kbps |
| IdOLA-Net | Jakarta | sales@idola.net.id | 17 cities 2Mbps | 512Kbps |
| Wasantara-Net | Bandung | ?@bandung.wasantara.net.id | 22 other cities | 19.2Kbps VSAT |
| Melsa-Net | Bandung | ?@melsa.net.id | - | 64Kbps |

There are currently sixteen (16) Internet Service Provider (ISP) have received government licensed - some of them already in operation as listed.

- The Top Level Domain (TLD) for Indonesia (ID) is currently co-ordinated by the University of Indonesia (UI) in Jakarta, at

- http://www.adfa.oz.au/indonesian/faq.tld-id.html

- Rahmat M. Samik-Ibrahim (PIC=RMS46)

- University of Indonesia, Computer Science Center

- Jl. Salemba Raya 4, POBOX 3442 - Jakarta 10002

- Attn.:	Network Manager

- Fax:	+62-21-310-2774

- E-mail: postmaster@ui.ac.id

- University of Indonesia (UI) is going to drop their responsibility as TLD-ID fairly soon in 1996. As soon as, ITB has installed their Ku-Band ground station to AI3 - ITB is willing to take the responsibility as TLD-ID.

- Indonesian Internet traffic is typically unbalanced between outbound-inbound traffic. The outbound traffic from Indonesia to Internet is typically in the range of 20-30% of the link capacity. Only one exception, Institute of Technology Bandung (ITB) traffic is a balance Inbound-Outbound.

- Current Computer Networking Status

- A brief overview of the IT business / awareness can be easily seen in the following graphs pertaining:

- Hardware Sales / Market Penetration in Indonesia.

- Software Sales / Market Penetration in Indonesia.

- Concultacy Services in Indonesia.

- The number shown is in Million US$ (source http://www.swa.co.id).

- Private Sector

- IT has been one of the major trust behind the sucess of many businesses in Indonesia, IT has the primary impact in financial businesses. The trade and business communities are gradually catch up with the IT development. Major IT & network users in private sector are at:

- Financial Institutions

- Trade & Business Institutions

- Media & Newspaper Industries

- Travel, Hotel & Tourist Industries

Large collection of URL-list of these institutions is electronically available upon request from the author. Most of the rapid progress in the Internet Precense of the commercial & business communities are helped by several Internet precense providers as listed in Table 2

Table 2. Indonesia Internet Presence Providers

| Indonesia Connect | http://www.indocon.com/ |
| --- | --- |
| Promindo Global Internet | http://www.promindo.com/ |
| Indonesia Business Center Online | http://indonesia.clever.net/ |
| Indonesia Media Network | http://www.imn.co.id/ |
| Indonesia Business Center Online | http://www.indobiz.com/ |
| AdWork! | http://www.adwork.co.id/ |
| Indonesia Net | http://www.indonesianet.com/index.htm |
| Galeri Informasi Indonesia | http://www.go-indonesia.com/ |

Local Information Technology Industry

Local computer (hardware & software) vendors are quite abundant in Indonesia. There are several major IT consultan firms, e.g, Anderson Consulting, IBM, Hewlett Packard and DEC. When it goes into a specific Internet & TCP/IP computer networking, not that many people around in Indonesia. Most of the real TCP/IP-ers in Indonesia are located at research & educational institutions with major mailing lists:

sysop-l@itb.ac.id

pau-mikro@ee.umanitoba.ca

As for computer technology, most of the computer used are PC-based, normally, 386 & 486 class. For some computing intensive applications, we use a more powerful computing equipments.

Telecommunication Infrastructure and Services

The Indonesian telecommunication infra-structure is basically can be divided into two main categories, namely:

- Ground / Terrestrial Segment (moving towards 2Mbps backbone).

- Satellite / Space Segment.

- In terms of Internet development we will look into both segments. Major operators in terrestrial segment are:

- PT. Telekomunikasi Indonesia (http://www.telkom.go.id). They are in process to put a B-ISDN and ATM Network at least in 4 cities in Indonesia.

- PT. Aplikanusa Lintas Arta (http://www.idola.net.id). They are in process to put a 2Mbps Frame Relay Network to integrate 17 citie in Indonesia.

- Several companies, such as, PT. Telkomsel, PT. Komselindo, PT. Pasific Satellite Nusantara are providing GSM & AMPS services.

- In Space segment, the situation is more interesting with several major operators / companies:

- PT. Elektrindo Nusantara, specially using TDMA VSAT (TCP/IP), DAMA & SCPC.

- PT. Citra Sari Makmur (CSM), specially using TDMA VSAT.

- PT. Aplikanusa Lintas Arta, specially using TDMA VSAT.

- PT. Compact Microwave Indonesia (CMI), specially using SCPC.

- IT Manpower Supply and Demand

- Computer technology is not an alien to many Indonesian with many computer colleges. Unfortunately, Internet & computer networking technologies are relatively new in Indonesia. There are only two major Indonesian Universities, i.e.,

- Institute of Technology in Bandung (ITB) (http://www.itb.ac.id/)

- University of Indonesia (UI) (http://www.ui.ac.id/)

- have the capacity to teach correctly the Internet technology. A training program would be crucial & strategic to support the Indonesian network development

- Government Sector

- A significant effort in the Indonesian government computerization can be found in the education, science and technology community. The monumental effort lead by several institutions, such as,

- Institute of Technology Bandung (http://www.itb.ac.id).

- The Science and Technology Networking IPTEK-NET (http://www.iptek.net.id/).

- The Indonesian National Research Council (http://www.drn.go.id/).

- Status by Ministry / Department

- Ministry of  Education & Culture

- Significant IT activities in many Indonesian universities is apparent. Users in Educational institutions contribute a significant portion to the Indonesian Internet users. More detailed description is described in the next section in educational & research networking activities.

- Ministry of Science and Technology

- Under the co-ordination of Ministry of Science and Technology (http://www.inn.bppt.go.id/), there are several government networking initiatives. Some of the major networking initiative under this ministry are:

- The Indonesian National Research Council (DRN)

- As part of the NRC effort to support their top-down and highly competitive research programs, the NRC has been supporting the Science & Technology Network (IPTEK-NET) as the major infra-structure for disseminate research results & encourage collaboration among Indonesian researchers.

- IPTEKnet provides several services such as:

- Scientific research and development activities in Indonesia.

- World-wide international access through the Internet.

- Library access.

- Primary Name Server for *.go.id.

- To performed its functions, several Web servers have been installed, namely,

- http://www.iptek.net.id/	The IPTEK-NET Web Server.

- http://www.drn.go.id/	The National Research Council Server.

- The connection to IPTEK-NET should be provided by each institutions and not by IPTEK-NET. IPTEK-NET is basically only maintain the gateway to InterNet.

- National Aerospace Agency (LAPAN).

- The Indonesian National Aerospace Agency (also known as LAPAN), is one of the pioneer in trying to acquire the networking technology specially in satellite & packet radio networking technology (their packet radio network is known as JASIPAKTA). They have collaborating with the German National Aerospace Agency (DLR).  Contact person of LAPAN JASIPAKTA is listed in Appendix II.

- Indonesian Science Institute (LIPI)

- The Indonesian Science Institute (LIPI) is the largest government own research and development agency. LIPI in Bandung is one of the most active group in building the computer network within LIPI. Some of LIPI’s  major Web servers are:

- http://www.lipi.go.id/		Main LIPI Home Page.

- http://www.inkom.lipi.go.id/	Home Page of Control & Computer 							Networking Group within LIPI.

- Network of Government Owned Strategic Industries.

- There is ten (10) government owned strategic industries & all of them will be networked. It is interesting to know that all of the strategic industries are not located in Jakarta. Only Bandung has the most dense strategic industries, namely,

- PT INTI (into.co.id), Indonesia's telephone digitalization pioneer, the center of excellence in telecommunication industry.

- PT IPTN (iptn.go.id), (Industri Pesawat Terbang Nusantara), Indonesia's sole manufacturers of aircrafts, the center of excellence in aircraft industry.

- PT. LEN (len.co.id), Indonesia’s microelectronics pioneer.

- PT PINDAD (pindad.go.id),  the center of excellence in weapon and munitions industry.

- A brief description of these strategic industries can be found at http://www.inn.bppt.go.id/government/bpis/bpis.html

- Ministry of Industry and Trade

- Ministry of Industry has been developing for several years the Center Data and Information Services. Web services are also provided by the Center at:

- http://idndprin.dprin.go.id/

- http://www.dprin.go.id/

- Information provided by the ministry of industry via their Web includes:

- PUSDATA OnLine Services (24hour BBS)

- Export-Import Industry, Industry Capacity, Tax Tariff, Industry Directory etc.

- Information for potential investors in Indonesia.

- Company profile of various industries in Indonesia.

- Export & Import status of industries in Indonesia.

- Vendor, Standard, Trade Statistics, OKI, G15, APEC, ASEAN, UNCTAD, and other foreign relation information (on CD-ROM etc).

- Industry Technology, Environment, Industrial R&D Centers

- The Policies & Profile in Metal, Machinery & Electronics Industries.

- Industrial Policies & Profile.

- Several group of Government own industries are currently looking at the possibilities to build their computer networking. These groups of companies are:

- The Oil companies, specially to co-ordinate among Oil companies. To name a few, these companies are Pertamina, Arco, Elnusa, Caltex.

- The Government Own Fertiliser Companies, specially to keep up with the demand. To name a few, these companies are PT. Pupuk Sriwidjaja, PT. Pupuk Kujang, PT. Pupuk Kalimantan Timur.

Ministry of Environment

Within the ministry of environment a collaboration with United Nation Development Program (UNDP) has created the Sustainable Development Networking Program (SDNP). It is intended to network four components, namely,

- Government institutions.

- Educational institutions.

- Commercial institutions.

- Non Government Organizations (NGO).

- Information is disseminated through  http://www.sdn.or.id/. Information specific to the Ministry of Environment is distributed through SDN at URL http://www.sdn.or.id/menlh/

- Other Departments

- Several other Indonesian government ministeries are looking more seriously into building their information system & networking. These ministry offices are Ministry of Forestry,  Ministry of Agriculture, Ministry of Finance & Taxation,  Ministry of Health and Ministry of Mining. A coordination among these ministries in implementing their information system is being sought by the National Development & Planning Agency.

- Educational & Research Network in Indonesia

- Educational & Research Network has been a major trust behind the development of computer network in Indonesia and can be traced back to early 1980. UUCP Network among Indonesian universities has been a pioneering work of University of Indonesia (ui.ac.id). Their UUCP system is still in operation - although, recently, they provide the service mostly for commercial & NGOs entities.

- Figure 3. Major University Network Topology (January 1996).

- It is interesting to note that only ITB is currently giving Internet access to most of other universities.

- Bandung is also of Interest in terms of Telecommunication and IT Research communities:

- The head office of, PT. Telekomunikasi Indonesia, the major Indonesia telecommunication industry, is located in Bandung.

- It was planned by the Ministery Post & Telecommunication, that in Bandung will be located a:

- Center of Excellence for Telecommunication Research

- It is in Bandung, the main human resources for major Indonesian telecommunication industries is produced by the two major universities, namely,

- Institute of Technology Bandung (itb.ac.id)

- Sekolah Tinggi Telekomunikasi (sttt.ac.id)

- In September 1995, at ITB, we have been successfully conducted an International Conference in Telecommunication Human Reseource Development.

- Bandung is the most dense location of the Indonesian government strategic industries, namely,

- PT. IPTN (iptn.go.id) - Indonesian Aircraft Company.

- PT. INTI (inti.co.id)- Telecommunication Equipment Industry.

- PT. LEN Industry (len.co.id) - Electronics & Semiconductor Industry.

- PT. PINDAD (pindad.go.id) - Ammunition & Armour Industry.

- Most of the industry has a direct connection LAN-to-LAN network to ITB to reach Internet & will increase to 2Mbps CDMA.

- Location of several major Indonesian private telecommunication manufacturing Industries:

- PT. CMI Compact Microwave Indonesia - Satellite Communication.

- PT. Telnic - Private Telecommunication Industry.

- Similarly with the other Industries, some of these private industries are also have a direct LAN-to-LAN connection with ITB using 2Mbps CDMA.

- Bandung is the location of most of technological related research institutions from many Indonesian ministeries.

- Future Trends and Recommendations

- Neutral Hub for Indonesian ISPs

- Total Web traffic in Indonesia is in the range of 80-100Mbyte/day. About 40-70% of the traffic are due to the request from *.id domain. This condition warrants a neutral hub among these providers. Major overseas requests are coming from three top domain, namely, *.com, *.edu and *.jp. Several efforts are in progress:

- One of the Indonesian international carrier PT. Indosat (http://www.indosat.go.id) is planning to put a single high-speed (more likely T1 or above) for one of the major Indonesian Internet entry point in Indonesia.

- A total of sixteen (16) Internet Service Providers have received their licensed from the Indonesian government. A neutral hub among these providers is one of the big issue in Indonesia. ITB has taken an initiative to realize the neutral hub among these providers using our 2Mbps CDMA MAN to connect them all. It is currently in negotiation process.

- Appropriate IT Infrastructure

- As mentioned earlier, the Information Technology (IT) infrastructure in Indonesia is adequate. Unfortunately, the distances in Indonesia is quite far and will cost a significant amount of funding to run a computer network covering a large area such as in Indonesia. Simple strategies can be implemented to reduce the required cost in building a public WAN in Indonesia, namely,

- Participation of private and government owned telecommunication service industries in providing a national data backbone.

- Share the existing communication infrastructure within most regional government offices with other institutions, including private sectors, educational institutions to reduce the cost.

- Application Development Methodology and Approaches

- There are several methodologies and approaches in building the computer networking in Indonesia. To name a few of the approaches are:

- Community-based Bottom Up Development, it is the most effective approach to be able to sustain the rapid development of computer network in Indonesia.

- Self-support and self-financing approach is used in most of the community-based development approach.

- Top down approach is related to funding and  required to improve the computer network quality and performance. However, a top-down approach requires a long process and justification for providing the required funding. To support such process, the result of community-based bottom-up would be very useful.

- IT Human Resources Training Strategies

- Given:

- The Indonesian government has aiming for a total of seven (7) million phone line to be implemented by the year 1997.

- A total of 75 government universities with 1,5 million trainable potential Internet users.

- A total of 1500 private universities with approx. 8-9 million trainable potential Internet users.

Strategy:

- Give a reduction / educational fee for university to access Internet would give a significant impact to the Indonesian Internet society.

- ITB has taken the lead in negotiation with various telecommunication service industries as well as ISP to give access to Indonesian universities. We hope this will give a significant impact to the society.

- One of the key factor in the success of the establishment of information society within an institution is the availability of engineering support team for answering any technical questions arises.

- Conclusions

- This report is an effort to identify various networking initiatives, current status & major recommendations. A significant increase in the Indonesian networking infrastructure as well as number of users is apparent in the 1995. We are looking at a total user of close to 20,000 in 1995. A total of sixteen (16) ISPs have received their licensed from the Indonesian government. Currently (end 1995) the total speed to Internet is 640Kbps, we are hoping to increase the speed to close to 5,4Mbps by the end 1996. ATM, B-ISDN & Frame Relay 2Mbps regional / nation wide backbone is currently in implementation & hope to be in operation by the end of 1996. Several TCP/IP-based VSAT networks are in implementation. A neutral hub among Indonesian ISPs is being proposed by ITB. Bandung, Indonesia will be an interesting place for telecommunication research and development with a government commitment for building a center of excelence in telecommunication research in Bandung in support the majority location of Indonesian strategic & telecommunication industries. Some of the activities & recommendations will reflect ITB’s commitment & view towards computer networking in Indonesia.

- Acknowledgment

- The author would like to thank many of his colleagues to enable the research of the Indonesian Networking Status. Special thanks goes to:

- The Rector and IT communities at Institute of Technology Bandung who have been monumental in keeping the network alive and running.

- United Nation ESCAP for providing the funding for the government computerization study.

- All member of sysop-l@itb.ac.id who has been providing the most information & spending many sleepless nights to keep the Indonesian network alive.

- Staff at IPTEK-NET (Mawan, Toto, Anto & Sahry) & INFO-RIS PT. Telkom (Rizkan & Asep) who has been helpful in providing the author Internet access.

- Member Library Policy Study Team of USAID Higher Education Development Support (HEDS), specially, Dr. Paul C. Parker & Dr. Richard Madaus who has been inspiring the author in viewing the information technology from other perspective.

- Prof. Dr. Harsono Taroepratjeka, the Director for Academic Facilities at the Ministry of Education and Culture.

- The member of the National Library Task Force.

Appendix I

A Glimpse of Various Indonesian Web Pages

a. Academic Web Pages

| Institute of Tech. Surabaya | http://www.telimek.lipi.go.id/its/its.htm |  |
| --- | --- | --- |
| Institute of Tech.Bandung (ITB) | http://www.itb.ac.id/ITB/itb.html |  |
| Institute of Tech. National (ITENAS) | http://www.iii.net/users/prasetyo/itenas.htm |  |
| University Parahyangan, Bandung | http://kappa.iis.u-tokyo.ac.jp/~fujimori/unpar/unpar.html |  |
| University Hasanuddin | http://www.mis.bppt.go.id/~arisoni/unhas/unhas.html |  |
| University Gajahmada | http://budi.ee.umanitoba.ca/gama/info.html |  |
| University Indonesia (UI) | http://www.ui.ac.id/ |  |
| Christian University Petra, Surabaya | http://www.petra.ac.id/ |  |
| University Padjadjaran | http://www.cs.unb.ca/grads/r6uy/unpad/unpad.html |  |
| University Sam Ratulangi | http://www.carleton.ca/~hmantiri/UNSRAT-MANADO.html |  |
| STMIK Gunadarma | http://www.cyberspace.org/~derrys/gundar.html |  |
| IUC on Microelectronics | http://www.itb.ac.id/ITB/iuc-me/iuc-me.html |  |
| Pangudi Luhur High School | http://web.mit.edu/user/r/h/rhandojo/PL/PL.html |  |
| Cendana High School | http://tam2000.tamu.edu/~aig0112/cendana.html |  |
| Indonesian Science and Technology Information Base | gopher://gopher.fedu.uec.ac.jp/11/.FEDUWIS/.Indonesia/ISTIB |  |

b. Government Web Pages

| Agency for Technology Assesment | http://www.inn.bppt.go.id/ |
| --- | --- |
| Indonesian Science Institute | http://www.lipi.go.id/ |
| PT. Indosat | http://www.indosat.go.id/ |
| PT. Telekomunikasi Indonesia | http://www.telkom.go.id/ |
| Bakosurtanal | http://fbgeo1.tuwien.ac.at:80/~famhar/bakos.html |
| Ministry of Public Work | http://www.pu.go.id/ |
| Ministry of Industry | http://202.46.245.35/ |
| Ministry of Health | gopher://radio-pc.bppt.go.id/11c%3a/kes |
| Ministry of Environment | http://www.sdn.or.id/menlh |
| Ministry of Tourism, Post & Telecommunication | http://www.telkom.go.id/PARPOSTEL/deppos.html |
| BPIS - Strategic Industry | http://www.inn.bppt.go.id/government/bpis/bpis.html |

c. Commercial & Business Web Pages

| Bank Bali - Direct Banking | http://www.indo.net.id/commercial/bbali/direct_banking.html |
| --- | --- |
| Garuda Indonesia City Check-In | http://www.inn.bppt.go.id/commercial/check-in.html |
| Indonesia Business Center Online | http://indonesia.clever.net/ |
| Jakarta Trade House | http://www.ib.com/jth/ |
| PT. Sarana Putra Makmur | http://www.iii.net/users/prasetyo/spm-1.html |
| List of Manufacturing Industries | http://202.46.245.35/doc/direk/indmenu.htm |

Appendix II

ITB’s connection summary

| Connected Institutions | Users | Connection |  | Plan 1996 | Remarks |
| --- | --- | --- | --- | --- | --- |
| ITB | 2400 | 10Mbps |  | 100Mbps ATM | Est. 8000 users in 1996. |
| Bandung CDMA MAN | 50 | 2Mbps |  | expand | Mostly Telecom Manufacturing Industries |
| Bandung 9600bps MAN | 300 | 9600bps VHF | expand |  | Universities |
| Bandung 1200bps MAN | 2000 | 1200bps VHF | expand |  | Universities, Research & High School |
| HF SSB 1200bps PSK | 200 | 1200bps | expand |  | Universities |
| Satellite SCPC 19.2Kbps | 300 | 19.2Kbps | 38.4Kbps connect to IPTEK |  | Telecom Indust. IPTEK in 1996 (19.2Kbps) |
| AX.25 1200bps to LAPAN | 50 | 1200bps | 9600bps |  | Connect to IPTEK (9600bps) |
| InfoRis | 200 | 19.2Kbps | 2Mbps |  | Connect to IPTEK (64Kbps) |
| IdOLA-Net | 500 | - | 64Kbps |  | 2Mbps Internet access |
| Sisfo-Net | 100 | - | will connect |  | Internet access |